MIND TBSS

(Created by Kayti Keith 3/19/19; Edited by Kayti Keith 5/7/19)

Scripts in: MIND_shared\_MIND\ResearchProjects\MRI_Processed\WM_Skeleton_TBSS\03_Analysis\Scripts

Processing times based on: Mac, FSL 5.09

Note: Most scripts require manual path changes to be done before they are run.

Step 1: File Setup

- Make a text ID file listing all subjects

- Run script A01_BET-1_Copy_b0_and_DKI_mets

  - This script sets up files for later processing

    - TBSS requires files to be in the folder format: /Your_Processing_Folder/Phase1/BETprocessing

Step 2: BET and BET QC

- Run script A02_BET-2_BET_and_QC

  - This script skull strips your B0 and metric maps

  - Enter your BET threshold

    - 0.3 was used for this project

- Run script A03_Pre-TBSS_MovingDKImaps-bet

  - This script arranges files into TBSS’s require folder structure:

    - /Your_Processing_Folder/Phase1/DKI

    - /Your_Processing_Folder/Phase1/WMM

Step 3: TBSS

- Run TBSS scripts: A04_TBSS_1 through A10_TBSS_5B

  - These scripts run through the entirety of TBSS and generate 4Ds and skeletonized 4Ds for all metrics:

Step 4: Split 4Ds

- Run script A11_4d_Split

  - This script splits 4D skeletonized maps into individual subj skeletons across all metrics

    - It also creates a copy of the JHU fornix into each subj folder for later processing